Megapixel HD IP Dual light Dome Camera

Model No.: XIP0228GW3ES

Main feature:

- Professional design for HD network camera, small, beautiful and delicate, best for home security.

- Material: aluminum alloy, innovative patented heat dissipation structure, make HD camera can works well under high temperature.

- 3.0 megapixel, Sony progressive scan

- Advanced digital signal processing technology, high brightness imported SMT IR source, show the best HD video effects in both day and night environment.

- 3.0 Megapixel HD CS lens+ IR-CUT auto switch in day and night,3.6mm,6mm,8mmfixed focus lens optional

- 2 IR LED and 2PCS white led, effective night vision distance is 25m, excellent performance in dark environment.

- Support 3D DNR, auto white balance, auto electronic shutter, auto electronic gain, real color reproduction, high resolution image.

- ONVIF protocol, can provide the SDK.

Optional back-end products for the system:

IE browse software, P2P HD remote monitoring management software (one key realize remote control without complicated settings), Embedded HD NVR, NVR control keyboard, CMS HD network surveillance platform management software, High-definition surveillance analysis software.

Technical parameters:

| Model: |  | XIP0228GW3ES |  |
| --- | --- | --- | --- |
| Camera | Imaging sensor: | 3.0MP,H.265+, MStar325+IMX317 ,1/2.9″ CMOS, Starlight |  |
|  | Scanning mode: | Progressive scanning |  |
|  | Illumination: | color 0.01Lux@F1.2,black/white:0.005Lux@F1.2, IR on 0 lux |  |
|  | Lens: | Standard with 3 megapixel 3.6mm,6mm,8mm fixed lens |  |
|  | Day and night switch: | IR-CUT dual optical filters auto switch in day and night |  |
|  | DNR: | 3D-DNR |  |
|  | IR source: | 2pcs IR Led, 2pcs white led |  |
|  | Effective night vision distance: | 25 meters (measured value without light condition) |  |
|  | P2P | YES |  |
|  | WDR | 128db |  |
|  | S/N Ratio | 50db |  |
|  | Audio pickup | Optional |  |
|  | Privacy mask | 1 |  |
|  | shutter speed | 1/(10-10000)s |  |
|  | FTP | YES |  |
| Standard compression | Image compression: | H.264 |  |
|  | Video compression: | H.265+/H.265/H.264, support dual-stream, AVI format; support stream 32Kbps ~ 3.5Mpbs adjustable;support CBR/VBR |  |
| Image | Max image size: | 3MP |  |
|  | Frame rate: | 3MP/2048*1536@25fps、1080P@30fps |  |
| Function | Interface protocol: | HTTP/RTSP/FTP/PPPOE/DHCP/DDNS/NTP/ONVIF2.6/,Hikvision private protocol |  |
|  | Intelligent alarm: | Motion detection, shadow alarm, network cable break, IP address conflict |  |
|  | POE | YES (optional),IEEE802.3at/af Standard PoE/IN:37-57V(15.4W), DC/OUT:12V/1A(12.95W) |  |
|  | Lightning Protection | Power, network, POE comprehensive lightning protection, in line with national standards GB/T17626.5, the international standard IEC61000-4-5. |  |
|  | features | Support WEB configure, support for OSD, support video transmission,support mobile surveillance, support for mobile detection alarm center after the screen pop-up reminders and linkage; supports JPEG capture; provide perfect the SDK development package; |  |
| Basic parameter | Power input: | DC 12V±10% |  |
|  | Power consumption: | 1W (IR off) / 9.5W (IR on) | 1.1W / 9.6W (IR on) |
|  | Work environment: | -40℃ ~ 85℃, humidity is less than 95% (no condensation) |  |